In this demonstration we will create a 3D sign in ArtCAM Express using some toolpath techniques, this can also be done in ArtCAM Insignia. We will start with simulating the pre-created toolpath so we can see the finished design, then we will create our own toolpaths on the model.

# Village Inn Sign

- Click on Open Model and select Village Inn.art.

- Click on the 3D View Tab found in the top left corner to switch to the 3D view.

- On the Project panel, click on plus sign next to the Toolpaths in the Project tree to highlighted the pre-created toolpaths, these are the toolpaths that we will now simulate to view the finished piece.

- Select each individual toolpath in sequence and right click it and select Simulate toolpath.

    - ArtCAM Insignia gives you the option to change the resolution of the simulation so you can view the finished piece more clearly where as ArtCAM Express has a fixed resolution.

- In ArtCAM Insignia select Simulate Toolpath (ArtCAM Express will do this automatically).

- Click on the 3D Graphics Option icon on the 3D View toolbar and under the Simulation Rendering select the wooden textures as a material to view the piece as it would look if cut in wood off a machine.

- Click on Apply and then Close

- The next steps will be creating the toolpaths over the design and saving the toolpaths out to a machine. But firstly we will delete the previously created ones.

- Right click on each of the toolpaths on the Project panel and select Delete.  Press Yes to confirm the delete.

- Right click on the toolpaths on the Project panel and select Delete Simulation

- Click on the the 2D View Tab to move back to 2D View

- In the 2D view select the vector as shown in the image.

- On the Project panel, click on Toolpaths to show the toolpath creation Function

- Select the Machine Relief function from the 3D Toolpaths section    located in the lower half of the panel.

- The Machine Relief panel automatically opens, use the following values:

  - Area to Machine – Selected Vectors

- To select the Finishing Options Tool click on the click to select…

- From Metric Tools

- Wood or Plastic

- 3D Finishing

- Select 3mm Ballnose

- Click on the Material Thickness down arrow and click Setup

- Material Thickness –  15mm

- Model Z Zero – Top

- Model Position in Material –  Top

- Click Ok

- Name –  Texture

- Click Calculate Now

- Click on the X to close the toolpath panel

- Click on the the 3D View Tab to move back to 3D View

- Select the Texture toolpath from the project panel, right click on it and select Simulate Toolpath.

- ArtCAM Insignia gives you the option to change the resolution of the simulation so you can view the finished piece more clearly, ArtCAM Express has a fixed resolution.

- In ArtCAM Insignia select Simulate Toolpath (ArtCAM Express will do this automatically).

- Click on the the 2D View Tab to move back to 2D View

- Select the 3 vectors as shown in the image below to do this select the first vector and hold down the shift key and select the final two vectors.

- On the Project panel, click on Toolpaths to show the toolpath creation functions

- Select the V-bit Carving function under the the 2D Toolpaths section  located in the lower half of the project panel.

- The V-Bit Carving function appears automatically, use the following values:

- Start Depth –  0

- Check the Limit Tool maximum Depth option

- Maximum Depth –  10mm

- Carving Tool – Click on Click to Select…

- Select V-Bit 32mm 90 degree tool from the Metric Tools- Wood or Plastic – V-Carving section of the database

- Click Calculate Now

- Click on the X to close the toolpath panel.

- Click on the the 3D View Tab to move back to 3D View

- Select the V – Bit Carving toolpath from the project panel, right click on it and select Simulate Toolpath.

- Click on the the 2D View Tab to move back to 2D View

- Select the very outer vector, select Toolpaths on the project panel & select the Profile toolpath function from the 2D Toolpaths section located in the lower half of the project panel.

- The profiling function appears automatically, use the following values:

- Profile - Outside

- Start Depth – 0

- Finish Depth – 12

- Profiling Tool – Click to Select…

- Select 12mm End Mill from the Metric Tools- Wood or Plastic – Roughing and 2D Finishing  section of the database

- Click on Calculate Now

- Click on the X to close the toolpath panel.

- Switch back to 3D View, select the calculated Profile toolpath on the project panel, right click on it and select Simulate Toolpath.